SPECIFICATION

Hippo-M MAX Connector

10mm PCB 2 Pins for COB/SMD

Basic Info

| Product Series | Hippo-M MAX |  |  |
| --- | --- | --- | --- |
| Item NO. | MX/2045-X10C-2P | MX/2045-B10C-2P | MX10BXB-L2C |
| Connection Type | Strip to Wire | Strip to Strip Joint | Corner Strip to Strip Bridge |
| With Wire or NOT | No wire | No wire | with wire (20AWG/0.52mm²) |

Physical Data

| Item NO. | MX/2045-X10C-2P | MX/2045-B10C-2P | MX10BXB-L2C |
| --- | --- | --- | --- |
| Shape | I shape, straight connection | I shape, straight connection | L shape, Corner connection |
| Size | 19*12.6*6.2 | 19.8*12.6*6.2 | 36.6*36.6*6.2 |
| Pin Quantity | 2 | 2 | 2 |
| Notch Width | 5.0mm | 5.0mm | 5.0mm |

Size detail, pls refer to below Dimensional Diagram

Material Data

| Housing Color | Transparent |
| --- | --- |
| Material | Polycarbonate(PC), Phosphor bronze |
| Surface Treatment | Nickel plating |
| Housing Deform Temperature | 80℃ |

Electrical Data

| Item NO. | MX/2045-X10C-2P | MX/2045-B10C-2P | MX10BXB-L2C |
| --- | --- | --- | --- |
| Voltage | DC3~48V | DC3~48V | DC3~48V |

Current & Temperature

| Item NO. | Max Current | PCB Copper Thickness Required | Temperature Raise (ΔT) | Max LED Strip Temperature T(strip) |
| --- | --- | --- | --- | --- |
| MX/2045-X10C-2P | 4A | 2 OZ | 10℃ | 65℃ |
|  | 5.0A | 3 OZ | 10℃ | 65℃ |
| MX/2045-B10C-2P | 3.0A | 2 OZ | 10℃ | 65℃ |
|  | 4.0A | 3 OZ | 10℃ | 65℃ |
| MX10BXB-L2C | 4A | 2 OZ | 10℃ | 65℃ |
|  | 5.0A | 3 OZ | 10℃ | 65℃ |

Note: Test with @TA=30℃ Wire Gauge=20AWG

Remark

- ΔT = Inner Temperature Raise for reference;

- T(strip) = temperature of working led strip

- In actual application, make sure T(strip) doesn't exceed the highest temperature listed above

( For example T(strip)<=60℃) otherwise lower the pcb current or apply extra cooling for long term working

Workable LED Strip

| Type | COB/SMD |  |  |  |
| --- | --- | --- | --- | --- |
| Size（WxH） | 10x3.9 mm |  |  |  |
| Light Color | MONO |  |  |  |
| IP Rating | IP20 / IP65 silicon |  |  |  |
| PCB Thickness | 0.25 ~0.35 mm |  |  |  |
| LED Density/ Meter | No Limited to COB，if SMD , it depends on SMD size |  |  |  |

Workable Wire Data

| AItem NO. | MX/2045-X10C-2P |  |  |  |
| --- | --- | --- | --- | --- |
| Wire Gauge | Best fit 18AWG/0.78mm² (PVC) & 18AWG/0.78mm² (Teflon) ; 20AWG/0.52mm² (PVC) |  |  |  |
| Outer Diameter | MONO OD＜2.1mm |  |  |  |

Working Environment

| Environment TEMP. | -10 ~ 45℃ |
| --- | --- |
| Environment | Indoor |

Dimensional Diagram

| Item NO.: | MX/2045-X10C-2P |  |
| --- | --- | --- |

| Item NO.: | MX/2045-B10C-2P |  |
| --- | --- | --- |

| Item NO.: | MX10BXB-L2C |  |
| --- | --- | --- |

Tolerance（mm）

| Size Range | Tolerance |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ≤3 | ±0.10 |  |  |  |  |  |
| ＞3≤10 | ±0.15 |  |  |  |  |  |
| ＞10≤20 | ±0.20 |  |  |  |  |  |
| ＞20 | ±0.25 |  |  |  |  |  |

If the diagram has marked the tolerance data, use it as a standard; If not , take the tolerance table as standard.